| Coefficients of Friction |  |  |  |
| --- | --- | --- | --- |
| Types of Surfaces | us, static | uk, kinetic | ur, rolling |
| Hardwood on Hardwood | 0.5 | 0.25 | - |
| Rubber on Dry Concrete | 1.0 | 0.75 | 0.04 |
| Steel on Steel (smooth) | 0.15 | 0.10 | - |
| Steel on Steel (oiled) | 0.05 | 0.03 | - |
| Steel Runners on Ice | 0.1 | 0.04 | - |
| Car Wheels on Rails | - | - | 0.0045 |
| Steel Ball Bearings on Steel | - | - | 0.0025 |

Lecture  - Friction - Sample Problems

1)	A wooden desk rests on a hardwood floor.  The desk weighs 250 N.  Make a force diagram, note all assumptions, estimate the coefficient of friction from the friction chart, and calculate the force required to:

a) just get the desk to move

b) keep the desk moving at constant velocity

c) move the desk by placing it on a cart with wheels

| FN F Ff Fg | Fg = - 250 N Assumptions FN = Fg No vertical motion F = Ff v = k | Ff = µFN Fs = µFg a) Fs = ? Fs = (0.5)(250 N) = 125 N b) Fk = ? Fk = (0.25)(250 N) = 62.5 N c) Fr = ? Fr = (0.04)(250 N) = 10 N |
| --- | --- | --- |

2)	A train with a weight of 120,000 N requires a force of 17,000 N to just start it moving and 11,000 N to keep it moving at constant velocity.  Make a force diagram and note all assumptions.  Calculate the coefficients of static and kinetic friction.

| FN F Ff Fg | Fg = - 120,000 N Fs = 17,000 N Fk = 11,000 N Assumptions FN = Fg No vertical motion F = Ff v = k | a) µs = ? Ff = µFN Fs = µs Fg µs = Fs / Fg = 17,000N / 120,000 N µs = 0.142 b) Fk = µk Fg µs = Fk / Fg = 11,000N / 120,000 N µk = 0.0917 |
| --- | --- | --- |